PRACTICAL SKILL STATION

Defibrillation and Cardiac arrest rhythm recognition

KEY TEACHING OBJECTIVES

Each candidate should receive practical instruction on the following:

1.	Cardiac arrest rhythm recognition

2.	The use of a defibrillator

- Capabilities

- Safety

- Delivery of a DC shock

EQUIPMENT REQUIRED

Resusci junior x 2

Monitor-defibrillator with paediatric pads x 2

IV cannula in situ

Heartsim and interface x 2

Syringes x 2

ENVIRONMENT

In two halves of the room set up the resuscitation manikin, heartsim, defibrillator and monitor.

PLAN

*Be positive and supportive throughout

Ensure accurate timing of the skills station - to ensure all candidates can attempt skill.

5 mins intro by instructors - giving time to go through monitor.

20 mins to allow all candidates to attempt skill (average three minutes per candidate)

5 mins conclusion

SET

"In this session I am going to teach you recognition of the some cardiac arrest rhythms and the safe use of a defibrillator.”

DYNAMIC RHYTHM RECOGNITION

Using the three cardiac arrest rhythms, demonstrate the principles of rhythm recognition.

DEFIBRILLATION – HANDS FREE

This station is taught using the 4-part technique described previously in the Pocket Guide to Teaching for Medical Instructors. The following techniques should be taught:

- Ensure CPR is continued

- Apply adhesive defibrillation electrode pads in correct positions

- Select energy required  100 J (4 J/kg) and charge defibrillator

- Briefly pause CPR to identify rhythm (VF)

State “stand clear” loudly and ensure rescuers are clear

If non-shockable rhythm, disarm the defibrillator, resume CPR immediately and follow the non-shockable arm of the protocol

- Deliver shock and recommence CPR immediately

- Continue CPR for 2 minutes

- Charge defibrillator near end of 2 minute cycle	(second instructor gives time warning such as “2 minutes has nearly elapsed”)

- Briefly pause CPR to simultaneously check for ROSC and assess rhythm (VF)

- ROSC is indicated by: normal breaths, cough in response to resuscitation breaths or presence of a central pulse – This must take no longer than 10 seconds. A pulse check should be performed, simultaneously to the rhythm assessment, as part of assessment for ROSC

- State “stand clear” loudly and ensure rescuers are clear

- Deliver shock and recommence CPR immediately (Monitor changes to sinus rhythm)

- Although monitor indicates sinus rhythm, the following steps are essential:

- Instructor to highlight importance of continuing CPR till end of 2 minute cycle even with apparent change of rhythm on monitor

- Assistant gives adrenaline 2.5ml of 1:10,000 (10 micrograms/kg) immediately after the second shock – this may be initiated by either candidate or instructor during testing

- Continue CPR for 2 minutes

- Charge defibrillator near end of 2 minute cycle	(second instructor gives time warning such as “2 minutes has nearly elapsed”)

- Briefly pause CPR to simultaneously check for ROSC and assess rhythm (sinus) - A pulse check needs to be performed to ensure ROSC.

- Disarm defibrillator

Steps 4 to 6 should occur in rapid succession and take no more than 10 seconds. Ensure that chest compressions and ventilations continue at all times except when the shock is being delivered.  Pauses for checking ROSC, rhythm, and delivery of shock must be no longer than 10 seconds. Safe defibrillation must also be demonstrated.

While there are additional steps in the shockable rhythm protocol, this skill station focuses on defibrillation.  Discussion should include giving amiodarone after the 3rd shock and doses of adrenaline every second loop.

Management of non-shockable rhythms is not demonstrated, but should be mentioned by the instructor. For example, “if after the rhythm check I observed a non-shockable rhythm I would disarm the defibrillator, recommence CPR immediately and follow the non-shockable arm of the protocol.”

DEFIBRILLATION - MANUAL

All APLS courses in Australia and New Zealand use hands-free defibrillators with pads.

If using paddles instead of pads the differences are:

- The paddles must be placed firmly on the chest over gel pads

- The paddles should only be charged on the chest. No CPR should be performed while charging.

- After delivery of the shock the paddles should be immediately returned to the defibrillator.

During this skill station correct pad selection, correct pad placement and the safety aspects of defibrillation should be emphasised:

## CORRECT PAD SELECTION

Standard adult automatic external defibrillators (AEDs) and pads are suitable for use in children older than 8 years. Ideally, for children between 1 and 8 years, paediatric pads with a paediatric capability should be used. Adult pads are acceptable if no paediatric pads are available

## CORRECT PAD PLACEMENT

The usual placement is antero-lateral. One pad is put over the apex in the mid-axillary line, and the other is placed just to the right of the sternum, immediately below the clavicle.

If the anterior-posterior placement is used, one pad is placed just to the left side of the lower part of the sternum, and the other just below the tip of the left scapula.

## SAFETY

A defibrillator delivers enough current to cause cardiac arrest.  The user must ensure that other rescuers are not in physical contact with the patient (or the trolley) at the moment the shock is delivered.  The defibrillator should only be charged when the paddles are in contact with the child and only discharged when in contact with the child or replaced properly in their storage position.

A high ambient oxygen concentration may lead to fire through “arcing”.  Any free-flowing oxygen (i.e. through a bag mask system) should be removed/turned off.

BASIC LIFE SUPPORT

It is also important to emphasise that basic life support must recommence immediately after defibrillation.  The new sequence will require particular attention to this point.  Emphasise that chest compressions and ventilations only stop for defibrillation and chest compressions stop for the rhythm check as the rhythm cannot be seen clearly on the monitor when chest compressions are ongoing.  If a drug is being given after a shock, chest compressions should be recommenced immediately after the shock and then the drug given.

RHYTHM CHECK

Make it clear that there is no need to check the rhythm at any other time than indicated in the algorithm. If the rhythm at that check has changed to an apparently perfusing rhythm then a pulse check should be done and if there is ROSC then post-resuscitation care given. If there is no pulse (or a pulse below 60 beats per minute with poor perfusion) and no other signs of life then use the PEA algorithm. If there is apparent asystole, check the leads and if confirmed use the asystole algorithm. This is all to prevent the cessation of chest compressions during cardiac arrest.

# TYPES OF DEFIBRILLATOR

Ensure that candidates are aware of the use of AEDs and their limitations in children. An adult AED can be used for a child over 8 years but for a child between 1 and 8 years paediatric attenuation pads or leads should be used. For infants of less than one year, a manual defibrillator which can be adjusted to give the correct shock is recommended.  However, if an AED is the only defibrillator available, its use should be considered, preferably with paediatric attenuation pads.  The order of decreasing preference for defibrillation in infants less than one year is as follows:

- Manual defibrillator

- AED with dose attenuator

- AED without dose attenuator

Many AEDs can detect VF/VT in children of all ages and differentiate “shockable” from “non-shockable” rhythms with a high degree of sensitivity and specificity.

Ensure that candidates are aware of the increasing prevalence of bi-phasic defibrillators and that current advice is to use them in the same doses as the older mono-phasic ones.

Advise candidates to familiarise themselves with the defibrillator in use at their own institution on their return to their place of work.

CLOSURE

Candidates should have the opportunity to formally ask questions. When these are answered the session can be closed by having the class repeat the system used for defibrillation.

ASSESSMENT TECHNIQUE

Candidates will be formally assessed during the testing station later in the course.